Establishing an Accredited Medical Equipment Calibration Laboratory

Manal Abdel Wahed, Sherif Ahmed Samy, Mohamed Montaser

Medical Equipment Calibration Lab, Faculty of Engineering, Cairo University

Abstract:

Accreditation is a means of determining the technical competence of laboratories to perform specific types of testing, measurement and calibration. Medical equipments require confirmation that they comply with specifications and safety regulations before they can be used in health care facilities. This paper presents guide lines to establish an Accredited Medical Equipment Calibration Laboratory in Egypt, based on the experience of the Medical Equipment Calibration Lab (MECL) of the Systems and Biomedical Engineering Department (SBME), Cairo University. A description of the calibration program and the requirements for calibration accreditation are explained. General overview of the MECL quality manual and other quality-related documentation is included.

Keywords: Accreditation; Calibration; EGAC; MECL.

1. Introduction:

Accreditation is a process through which programs, institutions, or systems are reviewed to ensure that pre-established uniform standards are met. It is common in many fields, including laboratories, hospitals, and schools. Accreditation is a useful tool for quality improvement and provides information to the facility or program, as well as to the public or users of the programs [1].

Any organization that performs measurements, calibrations or objective tests may seek accreditation, whether these activities are carried out in a permanent laboratory or on site.

Scopes of accreditation are documents that define specifically the measurements a laboratory is accredited to make. In addition the scope defines the ranges of the accredited measurand along with the associated best measurement capability expressed as an uncertainty for each measurand and range.

Calibration certificates should, wherever applicable, indicate the traceability to national or international standards of measurement and should provide the measurement result and associated uncertainty of measurement. Wherever applicable, and when suitable for customer requirements, a statement of compliance with identified metrological specifications can be accepted instead of measurement results and associated uncertainties. Only calibration certificates or reports endorsed by a recognized accreditation body's logo, or which otherwise makes reference to accredited status, are considered to satisfy traceability requirements. By definition, such an endorsed certificate or report will contain an appropriate statement of measurement results and/or a statement of compliance with an identified metrological specification accompanied by an appropriately defined uncertainty statement and a suitable statement of traceability.

Medical equipments require confirmation that they comply with specifications and safety regulations before they can be used in health care facilities. For this to be achieved, seeking accreditation according to an internationally acceptable standard, such as ISO/IEC17025 issued by International Organization for Standardization (ISO) and the International Electrotechnical Commission (IEC) [2], is an important step towards this goal. An accredited medical equipment calibration laboratory plays an important role in the health care quality system environment.

This paper deals with the accreditation of medical equipment calibration laboratories, based on the experience of the Medical Equipment Calibration Lab (MECL) of the Systems and Biomedical Engineering Department (SBME), Cairo University. The materials and methods section presents a description of the accreditation process and the requirements for calibration accreditation. A general overview of the MECL quality manual and other quality-related documentation is also included. The last section is the summary and discussion followed by the references section.

2. Materials and Methods:

2.1 Accreditation bodies:

There is an international accreditation body which leads all accreditation bodies world wide the International Laboratory Accreditation Cooperation (ILAC) [3]. Below the international accreditation body there are some regional accreditation bodies such as the European cooperation for Accreditation (EA) [4], the Asia Pacific Laboratory Accreditation Conference (APLAC) [5], and the Inter American Accreditation Cooperation (IAAC) [6]. In each country there is a national accreditation body such as the American Association for Laboratory Accreditation (A2LA) [7] in the United States of America, and the Egyptian accreditation council (EGAC) [8] in Egypt.

The Egyptian Accreditation Council (EGAC) operates under the terms of the Presidential Decree number 312/1996. EGAC is the sole Egyptian national body for the assessment and accreditation of all conformity assessment bodies undertaking certification, inspection and testing. EGAC accreditation is granted to laboratories that have shown that they meet, and continue to meet, the requirements of ISO/IEC 17025:2005, relevant ILAC guidelines and EGAC regulations.

2.2 EGAC requirements:

EGAC requirements for the competence of calibration and testing laboratories, ISO/IEC 17025, Section 5.6, contain requirements for measurement traceability and calibration. It has been prepared to ensure that laboratories comply with both the measurement and traceability requirements of ISO/IEC 17025 and the relevant requirements for calibration and testing  equipment not already covered by ISO/IEC 17025 that are set out in ISO 10012-1. The following are the main requirements.

- The Laboratory shall have a system for selecting, using, calibrating, checking, controlling and maintaining measurement standards, reference materials used as measurement standards and measuring/test equipment used in the performance of accredited calibrations and tests. Measurements performed by the Laboratory and any sub-contractors that it uses shall be covered by this system.

- The policies and procedures for this system shall be documented in the Laboratory's quality manual and associated quality documentation. They shall clearly define the responsibilities and duties of each member of staff involved in the activities listed in one of this document.

- The Quality Manager shall ensure that the measurement and calibration system used by the Laboratory is included in the program for quality system audits, and that the results of such audits are evaluated at the Laboratory's periodic review of the quality system.

- As required by ISO/IEC 17025: 5.2 and ISO/IEC 17025: 4.12, the Laboratory shall maintain records of training, competency, and staff authorized to use equipment and reference materials or to perform calibrations, tests, in-house calibrations and checks.

- The Laboratory shall use methods and procedures for the calibration of measuring equipment, reference measurement standards (including reference materials) and test equipment used in calibration and testing laboratories that comply with the requirements of ISO/IEC 17025: 5.4.

- The Laboratory shall maintain records for each item of measuring equipment, including reference measurement standards and reference material standards and test equipment, used in the performance of calibrations or tests. The records shall show that, either through in-house documentation or calibration certificates from external organizations, each calibration in the chain of traceability has been carried out.

- The Laboratory shall ensure that the records contain detailed information of the equipment/reference material used for calibrations, and that there is also a full and up-to-date history of the calibration of this equipment/reference material (ISO/IEC 17025: 5.5).

- The Laboratory shall have documented criteria for the selection of calibration intervals for all measuring and test equipment used.

For more details regarding this subject refer to [8].

2.3 EGAC regulations to be met by the laboratories:

These regulations apply to the accreditation of laboratories against ISO/IEC 17025 and any other relevant criteria of competence specified by EGAC. The term “laboratory” is deemed to cover both calibration and testing laboratories. The following are the main conditions.

- The Laboratory and its personnel shall be free from any commercial, financial and other pressures which might influence their technical judgment.

- The remuneration of the personnel engaged in calibration or testing activities shall not depend on the number of calibrations or tests carried out or on the results of such calibrations or tests.

- The Laboratory shall afford the Client or his representative reasonable cooperation to enable him to monitor the performance of the Laboratory in relation to his contract.

- The Laboratory shall afford EGAC and its representatives such reasonable accommodation and co-operation as necessary, to enable EGAC to monitor compliance with these Regulations and the relevant criteria of competence.

For more details regarding this subject refer to [8].

2.4 The accreditation process:

Any organization seeking accreditation may follow the following steps:

- Selecting the standard which it aims to follow and has accreditation based on, as an example ISO/IEC 17025 for calibration and testing laboratories.

- Start learning the standard to understand the different requirements stated in it. In this step the laboratory may ask the help (the support) of a consultant who is an expert in dealing with the ISO/IEC standard and all its requirements.

- Start building up the laboratory’s management system which will comply with the requirements of the standard. To do this step the lab assigns selected individuals with this job, this group of persons always named Quality Committee, which contains Quality manager, Quality officer, and may contain others, this depends on how large is the system which depends on the scope of the organization’s (laboratory’s) work. During building up the management system the Quality committee is supervised by the consultant.

- The quality manager do internal checks (internal audits) to make sure that the system Complies with the requirements of the standard.

- The laboratory starts to contact the accreditation body which is the EGAC.

- EGAC sends information pack to the laboratory.

- The laboratory submits the application form, a copy of the quality manual (On paper or in electronic form), and relevant associated documents.

- EGAC sends the Quality Manual to the Lead Assessor for review.

- Lead Assessor submits documentation review report to EGAC.

- EGAC conducts pre-assessment visit (if needed).

- EGAC submits document review corrective actions to the laboratory.

- EGAC appoints the relevant assessment team.

- EGAC conducts the initial assessment visit (s) to the laboratory.

- EGAC reviews the corrective actions to ensure the clearance of non-conformities (site visit maybe needed).

- Technical Advisory Committee reviews the clearance report and recommends accreditation.

- Accreditation is granted by the Accreditation Committee.

- Accreditation maintained through annual Surveillance visits and Re-assessment.

Figure 1 shows a flow chart for the accreditation process.

Fig.1 MECL accreditation process flow chart

2.5 The MECL quality manual overview:

The MECL Quality System for Calibration Services of medical equipment and devices is organized as follows:

- The first part, MECL Quality Manual (MECL QM) which contains the policies that describe the commitment of the staff with the quality system adopted, and Appendices that document the necessary information needed for it.

- The second part contains Managerial and Technical procedures established and maintained by MECL staff to meet their technical needs. In addition to Work Instructions of the calibration work that cover the scope of MECL.

The MECL managerial procedures include: Quality documents control, Review of customer requests, tenders and contracts, Customer care, Purchasing services and supplies, Record control, Control of nonconforming calibration work or results, Corrective/Preventive actions, Team communication, Internal audits, Management reviews.

The MECL technical procedures include: Identifying training needs and providing training of personnel, House keeping, Calibration work, Writing work instructions, Estimation of uncertainty, Handling, transport and storage of measuring equipments, Calibration of measuring equipments, Handling, transport and storage of test items, Issuing calibration certificates, Monitoring the validity of test results.

Figure 2 shows the MECL organization chart, and Figure 3 shows an example of the MECL calibration certificate.

Fig.2 MECL organization chart

Certificate No.:H04-AH005-0407

The here stated device was calibrated using standards with accuracy guidelines traceable to the National Institute of Standards & Technology (NIST) or have been derived from formulas of physical constants, or have been derived by ratio/ self calibration techniques. Evidence of traceability is on file at our service center.

| Client Name: |  |
| --- | --- |
| Client Address: |  |

| # Calibration Data |  |  |  |
| --- | --- | --- | --- |
| Calibration Eng.: | STE-06 | Test Date: | 11-04-2007 |
| Calibration Method: | MECL-WI-04 | Attachments | 3 pages |

| # Equipment Data |  |  |  |
| --- | --- | --- | --- |
| Equipment Type: | Pulse Oximeter Module |  |  |
| Model | 70450A1 | Serial No.: | 150096728 |
| Manufacturer: | BCI | Manuf. Year: | N.A |
| Location: | Neonatal | Prev. Calib.: | N.A |
| Date of receipt: | 11-04-2007 | Next Calib. On: | 11-04-2008 |
| Accessories: |  |  |  |
| Finger Probe Type: | Reusable | Manufacturer: | N.A |

| # Environmental conditions |  |  |  |
| --- | --- | --- | --- |
| Relative Humidity: | 41 % | Temperature: | 20.6oC |
| Test Location: |  |  |  |

Test results & uncertainties are stated in the attached test report.

This report should not be reproduced without written approval by

Medical Equipment Calibration Lab.

Page 1 of   4

Fig. 3 An example of the MECL calibration certificate

3. Summary and Discussion:

Accreditation is a formal recognition that a laboratory is competent to carry out specific tests or calibrations or types of tests or calibrations. There are some differences between certifications of quality assurance and accreditation. Actually, these certifications are two different, but, to some extent related concepts. For example, in the introduction part, ISO/IEC 17025 states that ‘‘testing and calibration laboratories that comply with this international standard will therefore also operate in accordance with ISO 9001 or ISO 9002’’. It also states that ‘‘certification against ISO 9001 and ISO 9002 does not, of itself, demonstrate the competence of the laboratory to produce technically valid data and results’’. The purpose of ISO 9000 series, on the other hand, is to describe fundamentals of quality management for organizations seeking advantage through the implementation of a quality management system [9]. Therefore, it will not be wrong to deduce that quality assurance and accreditation are two phenomena in which having a quality management system becomes possible without accreditation whereas accreditation becomes impossible without having a quality management system [10].

In summary, accreditation has become a means of demonstrating and documenting that a laboratory's fulfillment of the requirements of ISO/IEC 17025:2005 which means the laboratory meets both the technical competence requirements and management system requirements that are necessary for it to consistently deliver technically valid test results and calibrations.

Establishing an accredited medical equipment calibration laboratory in Egypt was clearly defined and structured in this paper. The accreditation for testing and calibration laboratories has many benefits which are:

- Establishes minimum competency standard.

- Identifies laboratory's specific competencies.

- Documents all non-conformities of the laboratory which is considered as deviation of the lab's management system from the requirements of the ISO/IEC standard or the regulations of the accreditation body.

- Improves quality of the laboratory's work.

- Enables continuous improvement.

- Assists user selection.

- Meets regulatory/procurement requirements.

- Assures acceptance of laboratory data.

- Breaks down barriers to international trade.

In conclusion; Laboratories are accredited, not registered or certified. Accreditation is an absolute necessity, at home and internationally. Accreditation facilitates world trade, by having reciprocity of data. Accreditation leads to more reliable results, and more credibility for our laboratories.

4. References:

- R. J. zabransky, "Coming soon to your area: state and local health agency accreditation", Clinical Microbiology Newsletter, vol. 28, no. 21, 2006.

- ISO/IEC 17025:2005 General Requirements for the competence of testing and calibration laboratories, International organization for standardization, Geneva, Switzerland.

- www.ilac.org.

- www.european-accreditation.org

- www.aplac.org

- www.iacc.org.mx/English/index.html

- www.a2la.org

- www.EGAC.gov.eg.

- ISO 9000:2005 Quality management systems: fundamentals and vocabulary. http://www.iso.org.

- E. Malkoc, W. Neuteboom, "The current status of forensic science laboratory accreditation in Europe", Forensic Science International, 167 (2007) 121-126.

تأسيس معمل معايرة أجهزة طبية معتمد

الإعتماد وسيلة لحساب القدرة التقنية للمعامل و ذلك لأداء أنواع معينة من الإختبارات و القياسات و المعايرة. الأجهزة الطبية تحتاج إلى توثيق على إمكانية استخدامها فى وسائل العناية الصحية. يقدم هذا البحث توجيهات لتأسيس معمل معايرة أجهزة طبية معتمد فى مصر، و ذلك بناءا على خبرة معمل معايرة الأجهزة الطبية الخاص بقسم الهندسة الحيوية الطبية و المنظومات بكلية الهندسة جامعة القاهرة. كما يقدم شرحا موصفا لبرنامج المعايرة و الإحتياجات الخاصة بالمعايرة المعتمدة. يحتوى البحث أيضا على نظرة شاملة لدليل الجودة الخاص بمعمل المعايرة، و بعض الأوراق ذات العلاقة بالجودة.